| Block of Code: | What it does: |
| --- | --- |
| # Packages imported into the project environment import pandas as pd import numpy as np import matplotlib.pyplot as plt import seaborn as sns #Clean style to make plots more legible sns.set_style('whitegrid') sns.set_palette('deep') | This code makes sure to input the necessary packages into the coding environment to make it usable for future lines of code. The style is also cleaned to make the code more legible |
| #### Loading the NHTS data in csv nhts_data = pd.read_csv('NHTS(in).csv') # Displaying Data to check that it was imported correctly nhts_data | This code loads the csv into code so it can be accessed, and then displays the data so you can check if the correct dataset was imported into the code |
| #This gives a quick summary of numerical variables nhts_data.describe() | This code block gives a quick summary of the numerical variables for the users information. |
| # This describes categorical values we are using in this project, for this project, we are using household income and make. nhts_data['make'].value_counts() | This code block gives value counts for the categorical values that we are using in this project, which for this project includes household income and make. |
| # This is in a separate cell so both are displayed nhts_data['household_income'].value_counts() | Household income is put in a separate code cell so it displays in the jupyter notebook |
| # Define the logical order for income (since strings sort alphabetically by default) income_order = [ 'Less than $10,000', '$10,000 to $14,999', '$15,000 to $24,999', '$25,000 to $34,999', '$35,000 to $49,999', '$50,000 to $74,999', '$75,000 to $99,999', '$100,000 to $124,999', '$125,000 to $149,999', '$150,000 to $174,999', '$175,000 or more' ] # Filter out the "I don't know" or missing values to clean data filtered_data = nhts_data[nhts_data['household_income'].isin(income_order)].copy() filtered_data['household_income'] = pd.Categorical(filtered_data['household_income'], categories=income_order, ordered=True) # Group and sort 'Make' by the total number of vehicles (descending) make_counts = filtered_data['make'].value_counts().index data_grouped = filtered_data.groupby(['make', 'household_income'], observed=False).size().unstack(fill_value=0) data_grouped = data_grouped.loc[make_counts] # Reorder rows based on popularity # Plotting data plt.figure(figsize=(16, 8)) palette = sns.color_palette("viridis", len(income_order)) # Using a perceptually uniform palette data_grouped.plot(kind='bar', stacked=True, ax=plt.gca(), color=palette, edgecolor='white', linewidth=0.5) # Cleaning up chart plt.title('Vehicle Make Distribution by Household Income (Sorted by Popularity)', fontsize=16, pad=20) plt.xlabel('Vehicle Make', fontsize=12) plt.ylabel('Number of Households', fontsize=12) plt.xticks(rotation=45, ha='right') # Angled labels are easier to read than vertical ones plt.legend(title='Household Income', bbox_to_anchor=(1.05, 1), loc='upper left') plt.grid(axis='y', linestyle='--', alpha=0.7) plt.tight_layout() plt.show() | This block of code orders the income ranges for clean data presentation, filters out data such as “I don’t know” or missing values, groups and sorts make by the total number of vehicles, and plots the data using a uniform palette, and then angles labels and cleans up the chart for effective data presentation |
| ## Step 1: tell Python you are making a figure and what size you want the figure to be plt.figure(figsize = (25,12)) ## Step 2: call the type of plot and tell the function what variables you want to plot boxplot = nhts_data.boxplot(column = 'household_size', by = 'make') ## Step 3: add customization to the plot ### this line rotates your labels on the x-axis by 90 so they are easy to read plt.xticks(rotation=90) ### the next two lines create your axis labels plt.xlabel('Make', fontsize=12) plt.ylabel('household_size (People)', fontsize=12) ### this line gives your plot a title plt.title('Boxplot of Household Size by Make', fontsize=14) plt.suptitle('') #removes the automatic pandas title ## Step 4: tell python to show the plot plt.show() | This box plot satisfies the box plot requirement, this block of code calls the type of plot and function, rotates the labels, and correctly creates the axis labels. |
| # Define the IDM acceleration function # The function should take the following inputs: # v : current speed of the follower vehicle (m/s) # s : current gap distance between leader and follower (m) # delta_v : speed difference = follower speed - leader speed (m/s) # v0 : desired velocity (m/s) # s0 : minimum spacing (m) # T : desired time headway (s) # a : maximum acceleration (m/s^2) # b : comfortable deceleration (m/s^2) # delta : acceleration exponent def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): # Step 1: Calculate the desired minimum gap s_star s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) # Make sure s_star is not negative s_star = max(s_star, s0) # Step 2: Calculate the acceleration using the IDM equation acceleration = a * (1 - (v/v0)**delta - (s_star/s)**2) return acceleration | This function is the IDM acceleration function, which does a lot of the simulation work. This part of the code returns the acceleration value from the IDM equation |
| # Test the IDM function with sample values # A follower traveling at 20 m/s, with a gap of 25 m, approaching the leader at 2 m/s # Define parameters v0 = 30 # desired velocity (m/s) s0 = 2 # minimum spacing (m) T = 1.5 # desired time headway (s) a = 1.0 # maximum acceleration (m/s^2) b = 1.5 # comfortable deceleration (m/s^2) delta = 4 # acceleration exponent # Test case test_v = 20 # follower speed test_s = 25 # gap distance test_delta_v = 2 # follower is faster than leader by 2 m/s # Call the function and print the result test_acc = idm_acceleration(test_v, test_s, test_delta_v, v0, s0, T, a, b, delta) print(f"IDM Acceleration Calculation Results: {test_acc}") # Think about: does this value make sense? The follower is approaching the leader, so should it accelerate or decelerate? | This tests the IDM function with samples values, which allows the user to see if the acceleration calculation results. |
| # Load the NGSIM data to get the leader vehicle trajectory ngsim_data = pd.read_csv('NGSIM.csv') # Select a trajectory pair trajectory_number = 6 data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number].reset_index(drop = True) # Extract the leader's position and speed over time time_data = data_subset['Time'].values leader_position = data_subset['leader_position(m)'].values leader_speed = data_subset['leader_speed(m/s)'].values # Identifying the time step in our data dt = 0.1 #seconds print(time_data) | This function loads the NGSIM data to get the leader vehicle trajectory. |
| # Set the IDM parameters v0 = 30 # desired velocity (m/s) s0 = 2 # minimum spacing (m) T = 1.5 # desired time headway (s) a_param = 1.0 # maximum acceleration (m/s^2) b = 1.5 # comfortable deceleration (m/s^2) delta = 4 # acceleration exponent # Initialize arrays to store simulation results n_steps = len(time_data) # number of simulation steps we will take, which will be equal to the length of the trajectory data sim_position = np.zeros(n_steps) # simulated follower position; we will create a vector with all zeroes as a start sim_speed = np.zeros(n_steps) # simulated follower speed; we will create a vector with all zeroes as a start sim_acc = np.zeros(n_steps) # simulated follower acceleration; we will create a vector with all zeroes as a start # Start at the real follower's initial state sim_position[0] = data_subset['follower_position(m)'].values[0] #this makes the starting value [0] same as the follower vehicle position sim_speed[0] = data_subset['follower_speed(m/s)'].values[0] #this makes the starting value [0] same as the follower vehicle speed | This code block sets the parameters for the simulation, initializes arrays to store the simulation results, and starts at the real followers initial state. |
| # Run the IDM simulation # We loop through each time step and calculate: # 1. The gap between the leader and follower # 2. The speed difference between the two vehicles # 3. The IDM acceleration # 4. The updated speed using: new_speed = old_speed + acceleration * dt # 5. The updated position using: new_position = old_position + speed * dt for i in range(n_steps - 1): # we do -1 since the first element is known (remember the step above) # Step 1 & 2: Calculate the gap and speed difference at this time step gap = max(leader_position[i] - sim_position[i], 0.1) delta_v = sim_speed[i] - leader_speed [i] # Step 3: Use the IDM function to calculate acceleration sim_acc[i] = idm_acceleration(sim_speed[i], gap, delta_v, v0, s0, T, a, b, delta) # Step 4 & 5: Update speed and position for the next time step sim_speed[i+1] = max(sim_speed[i] + sim_acc[i]* dt, 0) sim_position[i+1] = sim_position[i] + sim_speed[i]* dt print("Simulation complete!") | This code block runs the IDM simulation and tells the user that the simulation is complete. |
| # Plot: Acceleration vs Time — Comparing Leader, Real Follower, and IDM Follower plt.figure(figsize=(12, 6)) # Calculate Leader Acceleration (dv/dt) leader_speed_vals = data_subset['leader_speed(m/s)'].values leader_acc = np.diff(leader_speed_vals, prepend=leader_speed_vals[0]) / dt # Calculate Real Follower Acceleration (dv/dt) real_speed_vals = data_subset['follower_speed(m/s)'].values real_acc = np.diff(real_speed_vals, prepend=real_speed_vals[0]) / dt # Plot Leader Acceleration plt.plot(time_data, leader_acc, label='Leader (NGSIM)', color="red", linestyle='--', alpha=0.5) # Plot Real Follower Acceleration plt.plot(time_data, real_acc, label='Follower (NGSIM)', color="blue", alpha=0.6) # Plot Simulated Follower Acceleration (already calculated in Step 3 loop) plt.plot(time_data, sim_acc, label='Simulated Follower (IDM)', color="green", linewidth=2) # Formatting the Plot plt.xlabel('Time (seconds)') plt.ylabel('Acceleration (m/s²)') plt.title('Acceleration Comparison: Leader vs. Real vs. Simulation') plt.legend() plt.grid(True, alpha=0.3) plt.show() | This code block gives the acceleration vs time of the leader, follower, and simulated follower. |
| # Create a 2x2 subplot figure showing: trajectory, speed, acceleration, and gap for one vehicle pair # Step 1: Select a trajectory number and create the subset trajectory_number = 2 data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number] gap_distance = data_subset['leader_position(m)'].values - data_subset['follower_position(m)'].values time = data_subset['Time'].values # Step 2: Create the figure with 2 rows and 2 columns # hint: fig, axes = plt.subplots(2, 2, figsize=(14, 10)) fig, axes = plt.subplots(2, 2, figsize = (16, 10)) # Step 3: Top-left (axes[0,0]): Position vs Time for both leader and follower axes[1,1].plot(time, data_subset['leader_position(m)'].values, linewidth = 1.7, label = 'Leader', color = 'teal', alpha=0.5) axes[1,1].plot(time, data_subset['follower_position(m)'].values, linewidth = 1.7, label = 'Follower', color = 'orange', alpha=0.5) axes[1,1].set_ylabel('Position (m)', fontsize=11) axes[1,1].set_title('Position vs Time', fontsize=12) axes[1,1].legend(fontsize = 10) # Step 4: Top-right (axes[0,1]): Speed vs Time for both leader and follower axes[0,1].plot(time, data_subset['leader_speed(m/s)'].values, linewidth = 1.7, label = 'Leader', color = 'teal', alpha=0.5) axes[0,1].plot(time, data_subset['follower_speed(m/s)'].values, linewidth = 1.7, label = 'Follower', color = 'orange', alpha=0.5) axes[0,1].set_ylabel('Speed (m/s)', fontsize=11) axes[0,1].set_title('Speed vs Time', fontsize=12) axes[0,1].legend(fontsize = 10) # Step 5: Bottom-left (axes[1,0]): Acceleration vs Time for both leader and follower axes[1,0].plot(time, data_subset['leader_acc(m/s^2)'].values, linewidth = 1.7, label = 'Leader', color = 'teal', alpha=0.5) axes[1,0].plot(time, data_subset['follower_acc(m/s^2)'].values, linewidth = 1.7, label = 'Follower', color = 'orange', alpha=0.5) axes[1,0].set_ylabel('Acceleration (m/s^2)', fontsize=11) axes[1,0].set_title('Acceleration vs Time', fontsize=12) axes[1,0].legend(fontsize = 10) # Step 6: Bottom-right (axes[1,1]): Gap Distance vs Time axes[0,0].plot(time, gap_distance, linewidth = 1.7, label = 'Gap Distance', color = 'teal', alpha=0.5) axes[0,0].set_ylabel('Gap Distance (m)', fontsize=11) axes[0,0].set_title('Gap Distance vs Time', fontsize=12) axes[0,0].legend(fontsize = 10) # Step 7: Add title for the whole figure and adjust layout fig.suptitle('Vehicle Data for trajectory pair 2', fontsize = 30) plt.tight_layout() | This code block gives vehicle data for trajectory pair 2 such as gap distance vs time, speed vs time, acceleration vs time, and plots its into four subplots for easy digestion by the user. |